Reflection Report

on

Imitation of Television Remote Control System using STM32 MCU

| Student Credentials |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name |  | PRN |  | Contact No. |
| Aditi Patil |  | 202301070136 |  | 9545144544 |
| Kanak Panchal |  | 202301070135 |  | 93272 94119 |
| Krushna Yerawar |  | 202301070100 |  | 93598 03722 |
| Key Objectives: |  |  |  |  |
| - Implement Push-Button Controls: Enable channel switching, volume adjustment, and power ON/OFF using physical push-buttons. |  |  |  |  |
| - Display Channel on 7-Segment Display: Show the current channel number using a 7-segment display for quick visibility. |  |  |  |  |
| - Provide Status on LCD Screen: Display detailed system status like “TV ON,” “Channel 5,” and “Volume: 10” on an LCD. |  |  |  |  |
| - Simulate Power Control with Relay: Use a relay module to mimic the ON/OFF state of a television. |  |  |  |  |
| - Integrate Buzzer for Alerts: Generate sound alerts via a buzzer for power status changes and user interactions. |  |  |  |  |
| Intended Outcomes: |  |  |  |  |
| - Channel control using push-buttons with 7-segment display feedback. |  |  |  |  |
| - Volume adjustment and mute functions with real-time LCD updates |  |  |  |  |
| 3. Power ON/OFF simulation via relay and status display. |  |  |  |  |
| - Audio feedback through buzzer on every valid input. |  |  |  |  |
| 5. User-friendly interface with clear error messages and smooth control flow. |  |  |  |  |
| What real-world situation does this problem relate to? |  |  |  |  |
| -  Smart TV remotes that interact with TVs using embedded systems. |  |  |  |  |
| - Home automation systems where appliances (TVs, lights, fans) are controlled via microcontrollers |  |  |  |  |
| - User-interface development for embedded consumer electronics (using displays, buzzers, and buttons). |  |  |  |  |
| - Testing & prototyping remote-control-like behavior before final product development. |  |  |  |  |
| Why is this problem important or worth solving? |  |  |  |  |
| - Embedded Systems Practice: Helps develop practical skills in microcontroller programming, peripheral interfacing, and real-time system design |  |  |  |  |
| - Prototype for Automation: Serves as a foundation for developing smart home automation systems |  |  |  |  |
| - Career Skills Development: Builds hands-on experience with STM32, which is valuable for jobs in embedded systems, IoT, and electronics design. |  |  |  |  |
| - Hands-On Embedded Systems Learning It involves working directly with STM32, buttons, LCDs, buzzers, and relays. This practical experience builds strong embedded system skills beyond theory. |  |  |  |  |
| What assumptions are made about the scenario? |  |  |  |  |
| - User understands basic remote operations – It's assumed the user knows how to operate a TV using channel, volume, and power buttons. |  |  |  |  |
| - Push-buttons are the only input method – The system uses physical buttons, not IR, Bluetooth, or touchscreen controls. |  |  |  |  |
| - TV states are simulated, not real – The actual television is not controlled; a relay simulates ON/OFF state. |  |  |  |  |
| - Channel and volume ranges are predefined – The number of channels and volume levels are limited and fixed in the code. |  |  |  |  |
| - Power must be ON for other controls to work – Channel, volume, and mute actions are only allowed when the TV is ON. |  |  |  |  |
| - Feedback devices (LCD, buzzer, 7-segment) work correctly – It's assumed that all output components are functioning and properly interfaced with the STM32. |  |  |  |  |
| List of Components Used |  |  |  |  |
| Name of Component | Specs |  | Quantity | Input/ Output/ Processing Element |
| STM 32 F4110RB Nucleon Board | 32 bit micro controller |  | 1 | Processing Element |
| Push Button | Momentary contact, tactile switch |  | 5 | Input |
| Buzzer | 5V active piezo buzzer |  | 1 | Output |
| LCD 16*2 | 16 chars, 2 lines, HD44780 |  | 1 | Output |
| 7 Seg Display | Common cathode, 10-pin |  | 1 | Output |
| LED | 5mm, 2V, 20mA standard |  | 1 | Output |
| Jumper Wires | Male to male, male to female |  | 30 | Processing Element |
| System Architecture – Block Diagram |  |  |  |  |
| Configured Pin Diagram from STM32 CubeIDE |  |  |  |  |
| Mention all the settings required to configure peripherals in STM32 CubeIDE |  |  |  |  |
| Challenges faced |  |  |  |  |
| - STM32 Kit Unavailable: We had to switch platforms due to the unavailability of the STM32 kit. |  |  |  |  |
| - Damaged Components: Some sensors and connectors were not working properly, causing delays. |  |  |  |  |
| - Less Time: Limited hackathon duration made it difficult to test and refine the solution fully. |  |  |  |  |
| Photo Gallery |  |  |  |  |
| Appendices: |  |  |  |  |
| Firmware – Source STM32 CubeIDE |  |  |  |  |
| /* USER CODE BEGIN Header */ /** ****************************************************************************** * @file : main.c * @brief : Main program body ****************************************************************************** * @attention * * Copyright (c) 2025 STMicroelectronics. * All rights reserved. * * This software is licensed under terms that can be found in the LICENSE file * in the root directory of this software component. * If no LICENSE file comes with this software, it is provided AS-IS. * ****************************************************************************** */ /* USER CODE END Header */ /* Includes ------------------------------------------------------------------*/ #include "main.h" #include "i2c.h" #include "usart.h" #include "gpio.h" /* Private includes ----------------------------------------------------------*/ /* USER CODE BEGIN Includes */ #include "i2c_lcd.h" #include "stdio.h" /* USER CODE END Includes */ /* Private typedef -----------------------------------------------------------*/ /* USER CODE BEGIN PTD */ /* USER CODE END PTD */ /* Private define ------------------------------------------------------------*/ /* USER CODE BEGIN PD */ #define SEG_GPIO_PORT GPIOB #define SEG_A_PIN GPIO_PIN_0 #define SEG_B_PIN GPIO_PIN_1 #define SEG_C_PIN GPIO_PIN_2 #define SEG_D_PIN GPIO_PIN_12 #define SEG_E_PIN GPIO_PIN_13 #define SEG_F_PIN GPIO_PIN_14 #define SEG_G_PIN GPIO_PIN_15 /* USER CODE END PD */ /* Private macro -------------------------------------------------------------*/ /* USER CODE BEGIN PM */ /* USER CODE END PM */ /* Private variables ---------------------------------------------------------*/ /* USER CODE BEGIN PV */ uint8_t isOn = 0; uint8_t volume = 30; uint8_t channel = 1; uint8_t isMuted = 0; char displayBuffer[16]; const uint8_t segment_digits[10][7] = { {1,1,1,1,1,1,0}, // 0 {0,1,1,0,0,0,0}, // 1 {1,1,0,1,1,0,1}, // 2 {1,1,1,1,0,0,1}, // 3 {0,1,1,0,0,1,1}, // 4 {1,0,1,1,0,1,1}, // 5 {1,0,1,1,1,1,1}, // 6 {1,1,1,0,0,0,0}, // 7 {1,1,1,1,1,1,1}, // 8 {1,1,1,1,0,1,1} // 9 }; /* USER CODE END PV */ /* Private function prototypes -----------------------------------------------*/ void SystemClock_Config(void); /* USER CODE BEGIN PFP */ /* USER CODE END PFP */ /* Private user code ---------------------------------------------------------*/ /* USER CODE BEGIN 0 */ void display_digit(uint8_t digit) { if (digit > 9) return; // Only handle digits 0-9 HAL_GPIO_WritePin(SEG_GPIO_PORT, SEG_A_PIN, segment_digits[digit][0] ? GPIO_PIN_SET : GPIO_PIN_RESET); HAL_GPIO_WritePin(SEG_GPIO_PORT, SEG_B_PIN, segment_digits[digit][1] ? GPIO_PIN_SET : GPIO_PIN_RESET); HAL_GPIO_WritePin(SEG_GPIO_PORT, SEG_C_PIN, segment_digits[digit][2] ? GPIO_PIN_SET : GPIO_PIN_RESET); HAL_GPIO_WritePin(SEG_GPIO_PORT, SEG_D_PIN, segment_digits[digit][3] ? GPIO_PIN_SET : GPIO_PIN_RESET); HAL_GPIO_WritePin(SEG_GPIO_PORT, SEG_E_PIN, segment_digits[digit][4] ? GPIO_PIN_SET : GPIO_PIN_RESET); HAL_GPIO_WritePin(SEG_GPIO_PORT, SEG_F_PIN, segment_digits[digit][5] ? GPIO_PIN_SET : GPIO_PIN_RESET); HAL_GPIO_WritePin(SEG_GPIO_PORT, SEG_G_PIN, segment_digits[digit][6] ? GPIO_PIN_SET : GPIO_PIN_RESET); } /* USER CODE END 0 */ /** * @brief The application entry point. * @retval int */ int main(void) { /* USER CODE BEGIN 1 */ /* USER CODE END 1 */ /* MCU Configuration--------------------------------------------------------*/ /* Reset of all peripherals, Initializes the Flash interface and the Systick. */ HAL_Init(); /* USER CODE BEGIN Init */ /* USER CODE END Init */ /* Configure the system clock */ SystemClock_Config(); /* USER CODE BEGIN SysInit */ /* USER CODE END SysInit */ /* Initialize all configured peripherals */ MX_GPIO_Init(); MX_USART2_UART_Init(); MX_I2C1_Init(); /* USER CODE BEGIN 2 */ lcd_init(); /* USER CODE END 2 */ /* Infinite loop */ /* USER CODE BEGIN WHILE */ while (1) { /* USER CODE END WHILE */ /* USER CODE BEGIN 3 */ // main // Check ON/OFF button (PC13) if (HAL_GPIO_ReadPin(GPIOC, GPIO_PIN_13) == GPIO_PIN_RESET) { HAL_Delay(200); // debounce isOn = !isOn; HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, isOn ? GPIO_PIN_SET : GPIO_PIN_RESET); lcd_clear(); HAL_Delay(500); display_digit(0); } if (isOn) { // Channel UP (PA0) if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_RESET) { if (channel < 9) channel++; display_digit(channel); char msg[10]; int len = sprintf(msg, "CH: %d\n", channel); // convert channel to string HAL_UART_Transmit(&huart2, (uint8_t*)msg, len, HAL_MAX_DELAY); HAL_Delay(300); // debounce } // Channel DOWN (PA1) if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_1) == GPIO_PIN_RESET) { if (channel > 1) channel--; display_digit(channel); char msg[10]; int len = sprintf(msg, "CH: %d\n", channel); // convert channel to string HAL_UART_Transmit(&huart2, (uint8_t*)msg, len, HAL_MAX_DELAY); HAL_Delay(300); } // Volume UP (PA8) if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_8) == GPIO_PIN_RESET) { if (volume < 100 && !isMuted) volume++; HAL_Delay(300); } // Volume DOWN (PA9) if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_9) == GPIO_PIN_RESET) { if (volume > 0 && !isMuted) volume--; HAL_Delay(300); } // Mute toggle (PA4) if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_4) == GPIO_PIN_RESET) { isMuted = !isMuted; HAL_Delay(300); } // LCD Display lcd_put_cursor(0, 0); sprintf(displayBuffer, "VOL: %d ", isMuted ? 0 : volume); lcd_send_string(displayBuffer); lcd_put_cursor(1, 0); lcd_send_string(isMuted ? "MUTED " : "SOUND ON "); } else { lcd_put_cursor(0, 0); lcd_send_string("System OFF "); lcd_put_cursor(1, 0); lcd_send_string("Press ON Btn"); } } /* USER CODE END 3 */ } /** * @brief System Clock Configuration * @retval None */ void SystemClock_Config(void) { RCC_OscInitTypeDef RCC_OscInitStruct = {0}; RCC_ClkInitTypeDef RCC_ClkInitStruct = {0}; /** Configure the main internal regulator output voltage */ __HAL_RCC_PWR_CLK_ENABLE(); __HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1); /** Initializes the RCC Oscillators according to the specified parameters * in the RCC_OscInitTypeDef structure. */ RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI; RCC_OscInitStruct.HSIState = RCC_HSI_ON; RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT; RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON; RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI; RCC_OscInitStruct.PLL.PLLM = 8; RCC_OscInitStruct.PLL.PLLN = 64; RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2; RCC_OscInitStruct.PLL.PLLQ = 4; if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK) { Error_Handler(); } /** Initializes the CPU, AHB and APB buses clocks */ RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK¦RCC_CLOCKTYPE_SYSCLK ¦RCC_CLOCKTYPE_PCLK1¦RCC_CLOCKTYPE_PCLK2; RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK; RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV4; RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV2; RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1; if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK) { Error_Handler(); } } /* USER CODE BEGIN 4 */ /* USER CODE END 4 */ /** * @brief This function is executed in case of error occurrence. * @retval None */ void Error_Handler(void) { /* USER CODE BEGIN Error_Handler_Debug */ /* User can add his own implementation to report the HAL error return state */ __disable_irq(); while (1) { } /* USER CODE END Error_Handler_Debug */ } #ifdef USE_FULL_ASSERT /** * @brief Reports the name of the source file and the source line number * where the assert_param error has occurred. * @param file: pointer to the source file name * @param line: assert_param error line source number * @retval None */ void assert_failed(uint8_t *file, uint32_t line) { /* USER CODE BEGIN 6 */ /* User can add his own implementation to report the file name and line number, ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */ /* USER CODE END 6 */ } #endif /* USE_FULL_ASSERT */ |  |  |  |  |
| Proteus Circuit (if applicable) |  |  |  |  |
| “Paste your high-resolution image here” |  |  |  |  |
| Proteus Code (if applicable) |  |  |  |  |
| “Paste your complete tested code here” |  |  |  |  |